Introduction to MIPS:

MIPS ISA:

- Embark on a journey into the heart of how computers understand and work

- MIPS stands of Microprocessor without Interlock Pipeline

- Good architecture to start with Computer Language

- Has only two instructions that can access memory

- Also called REGISTER-REGISTER Architecture

- RISC (Reduced Instruction Set Computer):

  - Doing work with limited set of instructions

  - Not Efficient

  - But is small in size

  - Fast Execution and Efficient Pipeline

- CISC (Computer Instruction Set Computer):

  - Many instructions

ISA:

- Instruction Set Architecture

- The fundamental language that a computer processor understands

- Defines the set of instructions that the CPU can execute

Data Types (Acknowledged by QTSpim):

| Name | Size |
| --- | --- |
| Byte | 8-bit integer |
| Halfword | 16-bit integer |
| Word | 32-bit integer |
| Float | 32-bit floating-point number |
| Double | 64-bit floating-point number |

Overview of MIPS Machine:

Processor Registers:

- Features 32 general-purpose registers

- Denoted by $0 to $31

- Each register is 32 bits wide, capable of holding a single word of data

- Fastest storage available to the CPU

- Some temporary registers can be accessed by the users

Byte-Addressable Memory:

- MIPS memory is byte-addressable

- Means at every unique memory address 1 byte (8 bits) is stored

- Allows for flexible data manipulation from individual characters to larger data structures.

Instruction Execution Cycle:

- Both instructions and data reside inside memory.

- Vonn Newman Architecture

$t0 - $t7 & $t8 - $t9 (Most Commonly Used Registers)

00

00

00

2A

Byte Ordering:

- Based on the little-endian architecture

- LSB stored in the lowest memory address

- MSB stored in the highest memory address

- E.g.: 4210 in hex, word size is 0x0000002A

QTSpim Program Template:

- Initial header (“.text”, “.globl main”, “.ent main”)

- This is necessary otherwise the program won’t run

- I-Type Instruction Format:

  - Load, store, branch immediate instructions all use the I-type format.

| op (6 bits) | rs (5 bits) | rt (5 bits) | address (16 bits) |
| --- | --- | --- | --- |

  - R-type has all zeros in “op” and is identified by it

  - op, rs and rt always remains same

- Loading Larger Constants:

  - Can be loaded into a register 16 bits at a time

  - First loads the first 16 bits in hex code as lui

  - Then ORs (ori) and loads the lower 16 ones

  - Example:

    - 0000 0000 0011 1101 0000 1001 0000 0000

    - 0000 0000 0011 1101: lui: 0x0000003D

    - 0000 1001 0000 0000: ori: 0x00000900

- Address of instruction to be executed is stored in Program Counter

Extras:

- jr: Jump Register: Jumps back to initial position

- la: Load address

- lw: Load Word

- sw: Store word