1. Princeton (von Neumann) vs. Modern Day

Princeton architecture is the original "unified" design.

Similarities

- Stored-Program Concept: Both store the program instructions and the data in the same electronic memory.

- Fetch-Decode-Execute: Both follow the same fundamental cycle to process every command.

- Sequential Logic: Both use a Program Counter (PC) to track and execute instructions one after another by default.

Dissimilarities

- The Shared Bus: Princeton uses one path for both data and instructions (causing a bottleneck). Modern systems use multiple paths and "caches" to move both at once.

- Processing Speed: Princeton is strictly "one-at-a-time." Modern computers use Pipelining, where the CPU starts the next instruction before the current one is even finished.

- Single vs. Multi-Core: Princeton assumes a single processing unit. Modern computers have multiple "cores" (CPUs) on a single chip working in parallel.

2. Harvard vs. Modern Day

Harvard architecture is the "split" design.

Similarities

- Dual Pathways: Both use separate paths for instructions and data to increase speed.

- Parallel Access: Both can "read" an instruction and "read/write" data at the exact same clock cycle.

- Efficiency: Both are designed to avoid the "bottleneck" that slows down the Princeton model.

Dissimilarities

- Physical Separation: In pure Harvard, instructions and data are in completely different memory chips. In Modern Day, they are only separate inside the CPU (L1 Cache) but share the same RAM.

- Address Space: Harvard has two different "address maps." Modern Day usually has one unified address map for the entire system RAM.

- Flexibility: Pure Harvard is very rigid (hard to change programs). Modern Day is highly flexible, allowing the same memory space to be used for a video game one minute and a word document the next.

Summary "Cheat Sheet"

- Princeton: Everything shares one "pipe" (Simple but slow).

- Harvard: Instructions and Data have their own "pipes" (Fast but complex).

- Modern Day: A "Hybrid" that uses Harvard's speed inside the chip and Princeton's simplicity for the main RAM.